System Framework & Hardware integration

Robot Capabilities

• 	Person detection using ultrasonic sensor

• 	Greeting response

• 	Movement control: forward, backward, left, right

• 	Manual control via Android app (Wi-Fi)

• 	Automatic mode using proximity sensing

Hardware Integration

Wiring Overview

• 	ESP32 connects to:

• 	HC-SR04 (Trig/Echo pins)

• 	BTS7960 or L293D (motor driver inputs)

• 	Buzzer (digital output)

• 	Dual LED display (voltage/current feedback)

• 	Wi-Fi module (built-in)

• 	Power Supply:

• 	Use LM2596S to step down battery voltage to 5V

• 	AMS1117 for 3.3V devices (e.g., ESP32)

• 	Motors:

• 	Connect BTS7960 to high-current motors

• 	L293D for smaller motors or wheels

• 	Camera:

Optional Raspberry Pi camera connected to Pi board for streaming

Libraries

- WiFi.h or BluetoothSerial.h for connectivity

- Servo.h or motor control libraries

- NewPing.h for ultrasonic sensor

DFD (main controller)

System modes

Manual Mode via app

Automatic Mode (ULTRASONIC DETECTION)

Android App Integration

We have two Communication Options

- Wi-Fi: ESP32 connect to local network

Send HTTP POST Request (from app)

- URL: http: ip address of microcontroller //:5000/move

- Method: POST

- Form Data: dir=forward, backward, left, right)

To connect a React Native app to our Raspberry Pi robot over Wi-Fi, we use HTTP requests from the app to communicate with a Flask server running on the Raspberry Pi.

Architecture Overview

[React Native App] ⇄ [Wi-Fi Network] ⇄ [Raspberry Pi Flask Server] ⇄ [Robot Motors/Sensors]

Raspberry Pi Setup

THIS SETUP FOR MICROCONTROLLER

Install Flask:

pip3 install flask

- Create Flask Server (robot_server.py)

- Run the server

sudo python3 robot_server.py

- Find your Pi’s IP address:

hostname -I

Use this IP in your React Native app.

THIS SETUP FOR APP TO CONNECT ROBOT

React Native App Setup

- Install Axios for HTTP requests:

npm install axios

- Send Command to Raspberry Pi: here we have have to write code for sending http request by using pi ip address in (js)