## 1. Quick Visual Summary (Design & UI Overview)

Body: Compact trolley-style base with a vertical mast, 2–3 DOF articulated arm, and a front-facing touchscreen display.
Color Modes:

- Hospital Mode: Soft white and blue theme.

- Jewellery Mode: Matte black with gold accents and adaptive LED mood lighting.
Screen UI: Large interactive buttons — “Deliver,” “Collect,” “Security,” “Show Items” — plus a status bar (Battery | Mode | Alerts).
Indicator LEDs: Green = Ready | Amber(light yellow) = Busy | Red = Alert

## 2. Key Features (with Practical Logic)

### A. Pick & Place (Jewellery / Medicine Handling)

Purpose: Safely handle delicate items like jewellery or medicines.
Working Principle:
Camera detects the object → checks depth → adjusts gripper pressure → picks and places safely.
Hardware: RGB camera, IR depth sensor (e.g., Intel RealSense), and gripper with force sensor.
Pseudo Logic:

if detect_object(confidence > 0.7):

pose = compute_pick_pose(depth_map, camera_pose)

approach(pose, speed='slow')

while not force_sensor.triggered():

close_gripper(incremental=True)

lift_object(safe_velocity)

navigate_to(destination)

place_object()

### B. Autonomous Navigation & Obstacle Avoidance

Purpose: Navigate through crowded hospital corridors or busy jewellery shops.
Method: Uses SLAM for mapping and real-time obstacle avoidance with LIDAR, ultrasonic, and bump sensors.
Tech Stack: ROS + RTAB-Map/GMapping + DWA local planner.
State Machine Logic:

- States: IDLE → NAVIGATE → AVOID → INTERACT → RETURN

- NAVIGATE: Follow global plan from SLAM

- If obstacle < 0.8m → AVOID → slow down and replan

- If stuck > 10s → send alert to operator

### C. Voice, Touch & Gesture Control

Purpose: Enable natural and intuitive interaction.
How it Works:

- Mic array for speech input

- ASR (Vosk or Google Speech) for recognition

- Touch GUI on screen

- Basic gesture recognition (e.g., hand wave) via camera classifier
Example Flow:
“User: Bring patient file 23” → ASR → Intent Parsing → Map to task deliver_file → Enqueue → “Okay, bringing file 23.”

### D. Security & Monitoring

Purpose: Prevent theft and monitor restricted zones.
System: 360° camera with motion detection, face recognition (blacklist check), and auto alerts via MQTT/HTTP.
Security Logic:

if motion_detected and after_hours:

if face_unknown and loitering_time > 15:

trigger_siren()

send_alert(owner_phone, image_snapshot)

If suspicious activity is detected in jewellery mode, auto-lock drawers.

### E. Power Management & Self-Charging

Purpose: Continuous operation with smart charging.
Mechanism: Battery monitor + IR-guided docking station.
Logic:

if battery < 20%:

cancel_non_critical_tasks()

navigate_to(charging_dock)

align_using_ar_tags()

## 3. Component List (Practical Hardware Setup)

Compute: Raspberry Pi 4B (4GB) or NVIDIA Jetson Nano (for ML tasks)
Microcontroller: Arduino Nano (low-level control)
Navigation Sensors: LIDAR (RPLIDAR A2), Depth Camera (Intel RealSense D435), Ultrasonic (HC-SR04), IMU (MPU6050)
Manipulator: 2–3 DOF robotic arm with soft gripper & force sensor
Power & Communication:

- Li-ion Battery (24V, 20Ah) with BMS

- Docking Station (contact or pogo-pin based)

- Wi-Fi + optional 4G module
Other: 7” Touchscreen, Mic & Speaker Array, LED Mood Ring

## 4. Software Stack (Functional Modules)

- OS: Ubuntu + ROS (Noetic / Humble)

- Perception: OpenCV, TensorFlow Lite (Object & Face Detection)

- Navigation: SLAM (RTAB-Map / GMapping) + move_base

- Voice: Vosk (offline) or Google Speech API (online)

- Messaging: MQTT or REST for notifications

- UI: React-based web dashboard + Onboard Electron/Flask touchscreen app

## 5. Behavior Scenarios (Step-by-Step)

### Scenario 1 — Hospital: Medicine Delivery

- Nurse says, “Robot, deliver med box 12 to Ward B.”

- Robot confirms task and begins navigation.

- Navigates via SLAM, avoiding people and trolleys.

- On arrival: announces, “Medicine for Ward B — please confirm.”

- Nurse taps “Confirm” → Robot delivers and returns to base.

### Scenario 2 — Jewellery Shop: Show Diamond Ring

- Customer: “Show me ring model R-45.”

- Robot retrieves item from secured drawer.

- Displays rotating 3D view on-screen under soft lighting.

- For try-on, item is handed to staff (not directly to customer).

### Scenario 3 — Security Alert (After Hours)

- Motion detected near display at 2:00 AM.

- Robot zooms camera, performs face recognition.

- If unknown person detected → triggers siren, locks display, and sends image to owner.

## 6. Visual Flow Concepts (Easy for Diagrams)

Main Loop: Sense → Perceive → Decide → Act → Report
Pick & Place Flow: Detect → Localize → Plan → Grasp → Verify (via sensor) → Transport → Release → Confirm

(These can easily be converted into visuals using PowerPoint or Figma — I can also prepare a ready-to-use PPT template if you’d like.)

## 7. Pseudo-code Snippets (Simplified Examples)

Navigation Request Handler

def handle_navigation_request(destination):

map = load_slam_map()

goal = get_goal_coords(destination)

plan = planner.plan(map, goal)

while not at_goal():

local_obstacle = sensors.scan_local()

if local_obstacle:

avoid(local_obstacle)

follow_plan_step()

announce(f"Arrived at {destination}")

Battery Manager

def battery_manager():

while True:

pct = read_battery()

if pct < 15:

cancel_low_priority_tasks()

enqueue_task("go_charge")

sleep(30)

## 8. Safety & Compliance Guidelines

- Use soft grippers with force feedback to protect fragile items and humans.

- Include a physical emergency stop button.

- Ensure data privacy: store facial data securely with consent only.

- Implement human-in-loop confirmation for high-value handovers (e.g., jewellery).